Industrial Standard Operating Procedure (SOP)

Title: Conveyor Belt Integration System Using Mitsubishi FX5U PLC

Purpose

To define the operating procedure for the automated conveyor belt integration system controlled by the Mitsubishi FX5U PLC.

The system ensures precise, sequential movement of products detected by SICK retroreflective sensors and coordinated robotic pick-and-place operations.

Scope

This SOP applies to all automation technicians, operators, and maintenance personnel responsible for operating and maintaining the conveyor belt system integrated with the Mitsubishi FX5U PLC and SICK sensors.

Responsibilities

| Role | Responsibility |
| --- | --- |
| Operator | Operate the conveyor system as per this SOP; ensure proper placement of products near Sensor 1 only. |
| Automation Technician | Configure and troubleshoot PLC logic, sensors, and timing parameters. |
| Maintenance Team | Conduct routine maintenance, calibration, and cleaning of sensors and conveyor mechanisms. |
| Supervisor | Monitor system performance and verify adherence to this SOP. |

Equipment and Materials

- Mitsubishi FX5U PLC

- 4 SICK retroreflective sensors (S1, S2, S3, S4)

- Conveyor belt system

- SICK S300 safety sensor

- Emergency button

- THK linear slider

- ABB single arm robot

- AUBO i3 collaborative robot

- ABB dual arm robot

System Description

The system uses four retroreflective sensors placed on the side of the conveyor belt to control product movement:

| Sensor | Function |
| --- | --- |
| Sensor 1 (S1) | Detects product placement and initiates conveyor movement. |
| Sensor 2 (S2) | Stops the conveyor for ABB single arm robot to pick up the object. |
| Sensor 3 (S3) | Detects placement by ABB single arm robot and triggers conveyor movement after a 3-second delay. |
| Sensor 4 (S4) | Stops the conveyor for AUBO i3 collaborative robot to pick up the object. |

Operating Logic (Functional Sequence)

- Initial Conditions

- Ensure all sensors (S1–S4) are clear and no objects are on the conveyor

- Verify main switches, PLC and the robots are powered on

- Conveyor belt should be stationary

- Step-by-Step Operation (for one-by-one object movement)

- When S1 detects an object, the conveyor belt starts moving forward.

- Conveyor continues to move until Sensor 2 (S2) detects the object.

- Conveyor stops at S2.

- ABB single arm robot either rejects and places object on the counter if it is damaged or places it near Sensor 3 (S3) if it is accepted

- During this process, Conveyor remains stationary.

- Once S3 detects the placed object, the PLC initiates a 3-second delay timer (T1)

- After 3 seconds, conveyor starts moving again until the object reaches Sensor 4 (S4)

- Conveyor stops when S4 detects the object.

- AUBO i3 collaborative robot picks the object.

10)Once S4 becomes clear, PLC marks the position as empty and ready for the next object.

- Step-by-Step Operation (for one-by-one object movement)

- If a new object is placed at S1 while another is still at S2 or S4:

- The conveyor must not move

- The PLC counts the new object (increment counter).

- The conveyor only restarts once the preceding object has reached S3, and the 3-second delay after S3 has elapsed, and S4 is clear (object picked up by AUBO i3 collaborative robot).

- Invalid Placement Prevention

  - If an object is placed near S2, S3, or S4 without S1 being triggered, the conveyor remains stationary.

  - Operator must remove the incorrectly placed object before restarting.